Report

| Date: | 29/06/2020 | Name: | Safiya Banu |
| --- | --- | --- | --- |
| Course: | IIRS Outreach Program on Satellite Photogrammetry | USN: | 4AL16EC061 |
| Topic: | Introducing Photogrammetric Concepts | Semester & Section: | 8th B |
| Github Repository: | Safiya-Courses |  |  |

AFTERNOON SESSION DETAILS

Image of session

Report: Map: It shows an area as seen vertically form above. Different symbols and colours are used to represent various objects on a map. Aerial Photo: They are taken from an aircraft to show objects on the ground. They can be divided into vertical aerial photos and oblique aerial photos. Map; Aerial photo. Orthogonal projection; Central projection. Uniform scale; Variable scale. Terrain relief without distortion; Relief displacement. All objects represented on a particular scale; Only objects that are visible. Abstract representation; Real representation. WHAT IS PHOTOGRAMMETRY - The science of quantitative analysis of measurements from photographs - Photos - light - Gramma - to draw - Metron - to measure Distinct Areas in Photogrammetry Metric Photogrammetry - Making precise measurements from photos determine the relative locations of points. - Finding distances, angles, areas, olumes, elevations, and sizes and shapes of objects. Interpretative Photogrammetry - Deals in recognizing and identifying objects and judging their significance through careful and systematic analysis. Most common applications: - preparation of planimetric and Interpretation Sensing topographic maps - production of digital orthophotos - Military intelligence such as targeting BRANCHES OF PHOTOGRAMMETRY Based on platform: - Ground Based - UAV/drone based - Aerial Photogrammetry - Satellite Photogrammetry Based on processing techniques: Analogue System - Optical or mechanical instruments were used to reconstruct three-dimensional geometry from two overlapping photographs - The main product during this phase was topographic maps Digital System - Digital photogrammetry is applied to digital images that are stored and processed on a computer - Digital photogrammetry is sometimes called softcopy photogrammetry. - The output products are in digital form, such as digital maps, DEMs, and digital orthophotos saved on computer storage media. Analytic system - The computer replaces some expensive optical and mechanical components - Devices were analog/digital hybrids - Main developments- Analytical aerotriangulation, analytical plotters, and orthophoto projectors - Outputs - can be topographic maps, but can also be digital products such as digital maps and DEMs BASIC CONCEPT - The primary objective of the technique is to derive precise coordinates of a point - This is done by viewing the area from two different angles, thereby recreating the same conditions as it existed at the time of photography. TYPES OF AERIAL PHOTOGRAPHY - Vertical - Low oblique - High oblique Scale of Aerial Photography Before a photograph can be used as a map supplement or substitute, it is necessary to know its scale. On a map, the scale is printed as a representative fraction that expresses the ratio of map distance to ground distance, For example: RF=MD/ GD On a photograph, the scale is also expressed as a ratio, but is the ratio of the photo distance (PD) to ground distance. For example: RF PD/GD scale = f / H scale = photo distance + ground distance